SUPPLEMENTARY MATERIALS – ДОПОЛНИТЕЛЬНЫЕ МАТЕРИАЛЫ

SOIL POLLUTION AND EROSION PROCESSES IN THE REPUBLIC OF KAZAKHSTAN: REVIEW

ЗАГРЯЗНЕНИЕ ПОЧВ И ЭРОЗИОННЫЕ ПРОЦЕССЫ В РЕСПУБЛИКЕ КАЗАХСТАН: ОБЗОР

T. A. Paramonova, Y. A. Shynbergenov, D. V. Botavin, V. N. Golosov

Т. А. Парамонова, Е. А. Шынбергенов, Д. В. Ботавин, В. Н. Голосов

Eurasian Soil Science.

Почвоведение.

Appendix

T1

More than 6.3 million tons have been accumulated in the dumps of the spent Kurdai uranium deposit (Zhambyl region), which was operated in 1954-1965. m3 of uranium-containing waste [17]. At the same time, due to the reclamation of the territory carried out in 2005 with the overlap of surface sediments with the 1st layer of clay material, the exposure dose rate of gamma radiation at a height of 1 m is estimated as 0.14-0.95 mSv/hour. However, the specific activities of uranium and its radioactive decay products in the soils of the adjacent territory are significantly increased and reach 400 Bq/kg, 430 Bq/kg, 690 Bq/kg, respectively, for 238U, 232Th and 226Ra [15] with global average clarks in soils of 35 Bq/kg, 30 Bq/kg, 35 Bq/kg, respectively [20]. The volume specific activity of 222Rn passing from soils into the surface layer of the atmosphere is 20-90 Bq/m3 in open areas and 130-1300 Bq/m3 in houses of the village of Muzbel nearest to the deposit [17], while the average concentration estimates of 222Rn for residential buildings in the Republic of Kazakhstan do not exceed 10 Bq/m3 [20]. It is characteristic that the halos of soil pollution are complex – environmental risks arise in connection with man-made scattering of not only radionuclides in soils, but also such HM as As (up to 53 mg/kg), Cd (up to 1 mg/kg), Pb (up to 70 mg/kg), Cu (up to 91 mg/kg), Ni (up to 54 mg/kg), Cr (up to 66 mg/kg), Mo (up to 102 mg/kg). At the same time, a combined study of the content of radionuclides and HM in soils, plants, surface and groundwater of the territory reveals a high water migration ability and biological accessibility of elements of the geochemical association 234,238U - 226Ra – Cd – As – Ni against the background of relative immobilization of Pb, Cu and Cr compounds in soils.

Приложение

Т1

На отвалах отработанного Курдайского уранового месторождения (Жамбылская область), которое эксплуатировалось в 1954-1965 гг., накоплено более 6.3 млн. м3 уран-содержащих отходов [17]. При этом благодаря проведенной в 2005 г. рекультивации территории с перекрытием поверхностных отложений 1-м слоем глинистого материала, МЭДГИ на высоте 1 м оценивается как 0.14-0.95 мкЗв/час. Однако удельные активности урана и продуктов его радиоактивного распада в почвах примыкающей территории существенно повышены и достигают для 238U, 232Th и 226Ra соответственно 400 Бк/кг, 430 Бк/кг, 690 Бк/кг [15] при среднемировых кларках в почвах 35 Бк/кг, 30 Бк/кг, 35 Бк/кг, соответственно [20]. Объемные удельные активности 222Rn, переходящего из почв в приземный слой атмосферы, составляют 20-90 Бк/м3 на открытой местности и 130-1300 Бк/м3 в домах ближайшего к месторождению села Музбель [17], в то время как средние оценки концентрации 222Rn для жилых домов в РК не превышают 10 Бк/м3 [20]. Характерно, что ореолы загрязнения почв являются комплексными – экологические риски возникают в связи с техногенным рассеянием в почвах не только радионуклидов, но и таких ТМ как As (до 53 мг/кг), Cd (до 1 мг/кг), Pb (до 70 мг/кг), Cu (до 91 мг/кг), Ni (до 54 мг/кг), Cr (до 66 мг/кг), Mo (до 102 мг/кг). При этом сопряженное исследование содержания радионуклидов и ТМ в почвах, растениях, поверхностных и подземных водах территории выявляет высокую водно-миграционную способность и биологическую доступность элементов геохимической ассоциации 234,238U – 226Ra – Cd – As – Ni на фоне относительной иммобилизации в почвах соединений Pb, Cu и Cr.

Table S1. HM content in soils of a number of cities of the Republic of Kazakhstan (based on a review of publications)

Таблица S1. Содержание ТМ в почвах ряда городов РК (на основе обзора публикаций)

| The name of the settlement and the population | Number of sampling points | soil type by [9] / [8] | Heavy metals and their content in soils, mg/kg |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cr | Ni | Cu | Zn | Cd | Pb |
| MPC, mg/kg [7] |  |  | 6 | 4 | 33 | 55 | 0,05 | 32 |
| Almaty, 2 228 715 | 16 [12] 8 [26] | chernozems, foothill dark chestnut, chestnut, meadow chestnut, meadow gray / Endosalic Kastanozems, Kastanozems Gleyic, Kastanozems Chromic, Haplic Kastanozems | 51,65 [12] | 9,85d-12,87e [26] | – | 160,9 [12] 77,72f-93,50g [26] | 12-18 [12] 2,33h-2,39i [26] | 2,1-33,1b [10] |
| Shymkent, 1 222 055 | – | gray, takyric and gray-red-brown / Greyic, Takyric Greyic |  | – | – | – | 32 [12] | 37 [25], 441,9 [16], 6400-48000 [18] |
| Ust-Kamenogorsk, 354 770 | 2 [23] | haplic and gleyic chernozems [23] / Haplic and Gleyic Chernozems | – | – | 87,9-856,3 [23] | 625,3-2406,3[23] | 3,8l [12]-27,4k [23] | 187,2-1347,7 [23] 3870а [26] |
| Balkhash, 73 450 | – | desert grey-brown / Gypsisols | – | – | 137,9 [16] | 178 [16] | 8,9 [16] | 560,5 [16] |
| Kentau, 74 671 | 9 [5] 11 [13] | mountain gray lands [13] / Greyzemic Umbrisols | – | – | – 78-580 [5] 82,5 [13] | 120-920 [3] 256-3422 [5] 192-592 [13] | – – 1,651 [13] | 86-518 [3] 257-1742 [5] 147-462 [13] |

Note. Location of points and date of soil sampling: a) 1 km from the KazZinka facilities in Ust-Kamenogorsk; b) gas station; d) intersection of Tole Bi Avenue and Rozybakiev Street: 43°15'02"N 76°53'19 E, VI.2015; e) intersection of Abai Avenue and Maulenov Street: 43°14'30"N 76°56'10 E, XI.2015; f) intersection of Al-Farabi Avenue and Zharokov Avenue (Kazakhfilm district): 43°11'52"N 76°54'01 E, XI.2015; g) intersection of Ryskulov Avenue and Zhansugurov Street: 43°17’26" s.w. 76°56’13 v.d., VI.2015; i) upper h/w to Kaskelen (near the village of Dolan): 43°09’58"N 76°40’40 E, XI.2015; k) a residential area 1.5 km from the metallurgical plant; l) at a distance of more than 2 km from the plant

Таблица S2. Предельно-допустимые концентрации химических веществ в почвах РК [7]

Table S2. Maximum permissible concentrations of chemicals in the soils of the Republic of Kazakhstan [7]

| № | name of the substance | MPC (mg/kg) of the soil, taking into account the background (Clark) | limiting indicator |
| --- | --- | --- | --- |
| movable form |  |  |  |
| 1 | cobalt | 5,0 | general sanitary |
| 2 | fluorine | 2,8 | translocation |
| 3 | chromium | 6,0 | general sanitary |
| water-soluble form |  |  |  |
| 4 | fluoro | 10,0 | translocation |
| 5 | benz(a)pyrene | 0,02 | general sanitary |
| 6 | xylenes (ortho-, meta-, para) | 0,3 | translocation |
| gross shape |  |  |  |
| 7 | arsenic | 2,0 | translocation |
| 8 | ofu | 3000,0 | water and general sanitary |
| 9 | mercury | 2,1 | translocation |
| 10 | lead | 32,0 | general sanitary |
| 11 | lead + mercury | 20,0 + 1,0 | translocation |
| 12 | elemental sulfur | 160,0 | general sanitary |
|  | hydrogen sulfide | 0,4 | Air |
|  | sulfuric acid | 160,0 | general sanitary |
| 13 | styrene | 0,1 | Air |
| 14 | formaldehyde | 7,0 | -"- |
| 15 | potassium chloride | 560,0 | Water |

Table S3. MPC of the Republic of Kazakhstan and the actual average values of clarks of a number of Heavy Metals registered in cities of Kazakhstan (by population) (according to [1])

Таблица S3. ПДК РК и фактические осреднённые значения кларков ряда тяжёлых металлов, зарегистрированные в городах Казахстана (по численности населения) (по данным [1])

| Heavy metals | Cr | Cu | Zn | Cd | Pb |
| --- | --- | --- | --- | --- | --- |
| MPC for Kazakhstan, mg/kg (form) | 6 * | 3 * | 23 * | 0,5 | 32 ** |
| population / localities | 80 | 39 | 158 | 0,9 | 54,5 |
| >700,000 / Almaty | 82,7 | 55,1 | 201,1 | 2,9 | 66,2 |
| 300-700 000 / Aktobe, Astana, Karaganda, Kostanay, Pavlodar, Semey, Taraz, Uskaman, Shymkent | 55 | 30,1 | 115,8 | 0,8 | 45,6 |
| 100-300 000 / Aktau, Atyrau, Kyzylorda, Pavlodar, Petropavlovsk, Taldykorgan, Temirtau, Uralsk | 42,2 | 28,1 | 99,5 | 0,5 | 43,4 |
| <100,000 / Almaty region, Military, Karaganda region, Taldykurgan region | 81,5 | 28,2 | 92,4 | 1,4 | 39,5 |
| Village | 52,8 | 34,7 | 97,7 | 0,2 | 22,7 |

Note. * - mobile; ** - gross

Table S4. Radionuclide contamination of the soil cover of the territory of the Republic of Kazakhstan

Таблица S4. Радионуклидное загрязнение почвенного покрова территории РК

| Territory | Soil types |  | Pollution level | Source of pollution, year |
| --- | --- | --- | --- | --- |
|  | [9] | [8] |  |  |
| Koshkar-Ata tailings dump (Mangystau region) [14] | tailings dumps | Technosols | 41598.5*1010 Bq | uranium production |
|  |  |  | 800 Bq/kg | 226Ra |
| Semipalatinsk Test Site (STS) [6] | Solonets are automorphic Chestnut / Light Chestnut Light Chestnut salty Floodplain meadows | Haplic Solonetz Albic Haplic Kastanozems Chromic Haplic Kastanozems Sodic Umbric Fluvisols Oxyaquic | 18.5 ГBq/km2 | 137Cs, 1992 |
| Dolon village, ≈ 100 km to the West; village Kainar, ≈ 160 km to the left of the STS [6] | Floodplain meadows Dark Chestnut | Umbric Fluvisols Oxyaquic Haplic Kastanozems Chromic | 11 GBq/km2 |  |
| Saryzhal village, ≈ 120 km south of the STS [6] | Light chestnuts | Haplic Kastanozems Chromic | 15 GBq/km2 |  |
| Karaaul village ≈ 195 km south of the STS [6] | Light chestnuts | Haplic Kastanozems Chromic | 13.7 kBq/м2 |  |
| Shu River, province of the Shu-Sarysu uranium deposit [15] | Floodplain meadows Meadow-marsh | Umbric Fluvisols Oxyaquic Haplic Gleysols Dystric | 190-990 Bq/kg | 40К*, 2011 |
|  |  |  | 24-63 Bq/kg | 226Ra* 2011 |
|  |  |  | 29-68 Bq/kg | 228Ra*, 2011 |
| Almaty-Semey railway station (STS) 46 sites [24] to the East and South-east |  |  | 1000-3000 Bq/m2 | 2000-2001 |
| Almaty-Semey h/w (0-10) sections to the South-east and South [24] | Light chestnuts | Haplic Kastanozems Chromic | 3192 Bq/m2 / 34 Bq/kg | 137Cs, 2000 |
| Taldy Kurgan-Taskesken h/w (0-10) = 550 km to the Southeast [24] | Light chestnuts Chestnuts | Haplic Kastanozems Chromic | 3447 Bq/m2 / 30 Bq/kg | 137Cs, 2000 |
| Taldy Kurgan-Taskesken h/w (0-30) [24] |  |  | 7120 Bq/m2 | 137Cs, 2000 |
| Bakhty (0-10) [24] | Brown | Endosalic Calcisols Yermic | 3498 Bq/m2 / 32.6 Bq/kg | 137Cs, 2000 |
| Makhanshy-Bakhty h/w (0-28) [24] | Light Serozems | Greyic Phaeozems Albic | 4666 Bq/m2 / 15 Bq/kg | 137Cs, 2000 |
| Urjar (0-10) [24] | Meadow Chestnuts | Gleyic Kastanozems Chromic | 5021 Bq/m2 / 36.6 Bq/kg | 137Cs, 2000 |
| Urjar (0-30) [24] |  |  | 3554 Bq/m2 / 9.6 Bq/kg | 137Cs, 2001 |
| Taskesken (0-10) [24] | Chestnuts | Haplic Kastanozems Chromic | 3473 Bq/m2 / 28.6 Bq/kg | 137Cs, 2000 |
| Ayagoz-Georgievka h/w (0-10) [24] | Light Chestnuts salty | Haplic Kastanozems Sodic | 3331 Bq/m2 / 25.3 Bq/kg | 137Cs, 2000 |
| Georgievka (0-30) [24] | Chestnuts | Haplic Kastanozems Chromic | 3139 Bq/m2 / 8.6 Bq/kg | 137Cs, 2000 |
| Ayagoz-Georgievka h/w (0-10) [24] | Light Chestnuts, Chestnuts | Haplic Kastanozems Chromic | 297.8 Bq/m2 | 239,240Pu, 2000 |
| Ayagoz-Georgievka h/w (0-30) [24] |  |  | 216.1 Bq/m2 | 239,240Pu, 2001 |
| Georgievka (0-10) [24] | Chestnuts | Haplic Kastanozems Chromic | 308.3 Bq/m2 | 239,240Pu, 2000 |
| Georgievka (0-30) [24] |  |  | 677.1 Bq/m2 | 239,240Pu, 2000 |
| Kokpekti (0-10) [24] | Dark Chestnuts | Haplic Kastanozems Chromic | 234.8 Bq/m2 | 239,240Pu, 2000 |

Note. STS – Semipalatinsk test site; * - Gamma–emitting radionuclides; h/w – highway; (0-10), (0-28), (0-30) – The depth of the soil sample, cm.

Table S5. Costs of soil erosion in monetary terms for the Republic of Kazakhstan, a number of other countries and the world as a whole

Таблица S5. Издержки от эрозии почв в денежном эквиваленте для РК, ряда других стран и в целом в мире.

| Country | Damage, billion US dollars/year |
| --- | --- |
| Kazakhstan | 0.8 [11] |
| Russia | 9.7 [4] |
| Canada | 3.0 [21] |
| USA | 44.0 [19, 22] |
| in the world | 400.0 [2] |

Список литературы

- Алексеенко В.А., Алексеенко А.В. Химические элементы в геохимических системах. Кларки почв селитебных ландшафтов. Издательство Южного федерального университета, 2013. 380 с.

- Борьба с эрозией почв. Международное агентство по атомной энергии. https://www.iaea.org/ru/temy/borba-s-eroziey-pochv.

- Вестник Казгидромета. Экологический информационный бюллетень Республики Казахстан за 2017 год. РГП «Казгидромет». МЭ РК. 2017

- Глушко А.Я. Влияние водной и ветровой эрозии на земельный фонд юга европейской части России // Известия Дагестанского государственного педагогического университета. Естественные и точные науки. 2010. №. 1. С. 75-85.

- Джунусбеков М.М., Акбасова А.Д. Экологическая оценка уровня загрязненности почв города Кентау тяжелыми металлами // Гидрометеорология и экология. 2020. №. 3 (98). С. 34-43.

- Дубасов Ю.В., Пахомов С.А., Шагин Б.О. Радиационная обстановка вокруг Семипалатинского полигона и прилегающих поселках // Вестник НЯЦ РК. 2003. №. 3. С. 23-28.

- Об утверждении Гигиенических нормативов к безопасности среды обитания // Приказ МЗО РК от 21.04.2021, № ҚР ДСМ -32. Зарегистрирован в МЮ РК 22 апреля 2021 года № 22595

- Рабочая группа IUSS WRB. 2015. Мировая реферативная база почвенных ресурсов 2014, исправленная и дополненная версия 2015. Международная система почвенной классификации для диагностики почв и создания легенд почвенных карт. Доклады о мировых почвенных ресурсах №106. ФАО, Рим.

- Успанов У.У. (отв. ред). Почвенная карта Казахской ССР. М-б 1:2 500 000. М.: ГУГК, 1976.

- Aiman N., Gulnaz S., Alena M. The characteristics of pollution in the big industrial cities of Kazakhstan by the example of Almaty // J. Environ. Health Sci. Engineer. 2018. V. 16. P. 81–88.

- Almaganbetov N., Grigoruk V. Degradation of Soil in Kazakhstan: Problems and Challenges. In: Simeonov, L., Sargsyan, V. (eds) Soil Chemical Pollution, Risk Assessment, Remediation and Security. NATO Science for Peace and Security Series. Springer, Dordrecht. 2008. P. 309-320. https://doi.org/ 10.1007/978-1-4020-8257-3_27

- Baubekova A., Akindykova A., Mamirova A., Dumat C., Jurjanz S. Evaluation of environmental contamination by toxic trace elements in Kazakhstan based on reviews of available scientific data // Environ. Sci. Pollut. Res. 2021. V. 28. P. 43315–43328. https://doi.org/ 10.1007/s11356-021-14979-z

- Junusbekov M.M., Akbasova A.D., Seidakbarova A.D., Koishiyeva G.Zh., Sainova G.A. Ecological assessment of soil contamination by heavy metals affected in the past by the lead–zinc mining and processing complex in Kentau, Kazakhstan // Environ. Monit. Assess. 2023. V. 195 P. 586. https://doi.org/ 10.1007/s10661-023-11189-7

- Kadyrzhanov K.K., Lukashenko S.N. Radioactivity in Kazakhstan. Cases and Consequences. In: Birsen, N., Kadyrzhanov, K.K. (eds) Environmental Protection Against Radioactive Pollution. NATO Science Series. 2003. vol. 33. Springer, Dordrecht. https://doi.org/ 10.1007/978-94-007-0975-1_2

- Matveyeva I., Jaćimović R., Planinšek P., Stegnar P., Smodiš B., Burkitbayev M. Assessment of the main natural radionuclides, minor and trace elements in soils and sediments of the Shu valley (near the border of Kazakhstan and Kyrgyzstan) // Journal of Radioanalytical and Nuclear Chemistry. 2014. V. 299. P. 1399-1409. https://doi.org/10.1007/s10967-013-2902-3

- Ramazanova E., Lee S.H., Lee W. Stochastic risk assessment of urban soils contaminated by heavy metals in Kazakhstan // Science of The Total Environment. 2021. V. 750. P. 141535. https://doi.org/10.1016/j.scitotenv.2020.141535

- Salbu B., Burkitbaev M., Strømman G., Shishkov I., Kayukov P., Uralbekov B., Rosseland B.O. Environmental Impact Assessment of Radionuclides and Trace Elements at the Kurday U Mining Site, Kazakhstan // J. Environ. Radioact. 2013. 123. 14–27. https://doi.org/10.1016/j.jenvrad.2012.05.001.

- Saparov A. Soil Resources of the Republic of Kazakhstan: Current Status, Problems and Solutions. In: Mueller, L., Saparov, A., Lischeid, G. (eds) Novel Measurement and Assessment Tools for Monitoring and Management of Land and Water Resources in Agricultural Landscapes of Central Asia. Environmental Science and Engineering). Springer, Cham. 2014. https://doi.org/ 10.1007/978-3-319-01017-5_2

- Shojaeezadeh S.A., Al-Wardy M., Nikoo M.R., Mooselu M.G., Alizadeh M.R., Adamowski J.F., ... Gandomi A.H. Soil Erosion in the United States. Present and Future (2020-2050) // arXiv preprint arXiv:2207.06579. 2022. https://doi.org/10.48550/arXiv.2207.06579

- Stegnar P., Shishkov I., Burkitbayev M., Tolongutov B., Yunusov M., Radyuk R., Salbu B. Assessment of the Radiological Impact of Gamma and Radon Dose Ratesat Former U Mining Sites in Central Asia // J. Environ. Radioact. 2013. 123. 3–13. https://doi.org/10.1016/j.jenvrad.2012.12.005.

- van Vliet L.J.P., Junkins B.B., Gill B.R., Heigh B.L. The risk of water erosion indicator for Canada: integrating science and policy. ISCO 2004. 13th International Soil Conservation Organisation Conference – Brisbane, July 2004 Conserving Soil and Water for Society: Sharing Solutions

- Weltz M.A., Huang C.H., Newingham B.A., Tatarko J., Nouwakpo S.K., Tsegaye T. A strategic plan for future USDA Agricultural Research Service erosion research and model development // Journal of Soil and Water Conservation. 2020. V. 75. No. 6. P. 137A-143A. https://doi.org/10.2489/jswc.2020.0805A

- Woszczyk M., Spychalski W., Boluspaeva L. Trace metal (Cd, Cu, Pb, Zn) fractionation in urban-industrial soils of Ust-Kamenogorsk (Oskemen), Kazakhstan—implications for the assessment of environmental quality // Environ. Monit. Assess. 2018. V. 190. P. 362. https://doi.org/ 10.1007/s10661-018-6733-0

- Yamamoto M., Hoshi M., Takada J., Sakaguchi A., Apsalikov K.N., Gusev B.I. Current levels and distribution of 137Cs and Pu isotopes in soil on the Kazakhstan territory of the Kazakhstan-Chinese border: Semipalatinsk and Lob Nor nuclear test sites detonation // Journal of Radioanalytical and Nuclear Chemistry 2004. V. 261. P. 533–545. https://doi.org/ 10.1023/B:JRNC.0000037093.74415.7e

- Zhanibekov A, Issayeva R, Golovatyi S., Taspoltayeva A., Aitimbetova A., Nurtayeva A., ... Tulbasiyeva A. Assessment of Soil Contamination by Heavy Metals: A Case of Turkistan Region // Polish Journal of Environmental Studies. 2022. V. 31. No 2. P. 1985-1993. https://doi.org/ 10.15244/pjoes/142613

- Zholdassov O.E., Yelikbayev B.K., Umbetaliyev N.A., Erol O. The level of soil contamination with heavy metals in Almaty, Kazakhstan // Ecol. Environ. Conserv. 2016. V. 3. P. 1523-1527.